- Introduction to Addressing Theory

Q.1  List the key differences between IPv4 and IPv6 addressing by creating a two-column table with at least 4 points for each?

Answer

| IPv4 | IPv6 |
| --- | --- |
| - Uses 32-bit addresses | Uses 128-bit addresses. |
| - Example: 192.168.1.10 | Example: 2001:db8::10 |
| - Supports about 4.3 billion addresses | Supports a very large number of addresses |
| - Written in decimal numbers separated by dots | Written in hexadecimal numbers separated by colons |
| - Commonly uses NAT because of limited addresses | Usually does not require NAT because of the large address space |
| - Example private range: 192.168.x.x | Example unique local range: fd00::/8 |

Q.2  Given the following scenario: You have 4 devices (laptop, phone, smart TV, and printer) connected to your home WiFi. Assign a unique static IPv4 address to each device using the 192.168.10.0/24 network, and write down the IP, subnet mask, and default gateway for each device?

Answer

- Static IPv4 Addresses for Home Devices

Given Network: 192.168.10.0/24

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.10.1

| Device | Static IP Address | Subnet Mask | Default gateway |
| --- | --- | --- | --- |
| 1. Laptop | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| - Phone | 192.168.10.20 | 255.255.255.0 | 192.168.10.1 |
| - Smart TV | 192.168.10.30 | 255.255.255.0 | 192.168.10.1 |
| - Printer | 192.168.10.40 | 255.255.255.0 | 192.168.10.1 |

- Each device has a unique IP address, while all devices use the same subnet mask and default gateway.

Q.3  Calculate the number of valid subnets and hosts per subnet if you use a /27 CIDR subnet mask on a 192.168.1.0 network. List the first three valid subnet address ranges (network address to broadcast address) and the usable host IPs in each?

Answer

- Subnetting 192.168.1.0 Using /27

Given Network: 192.168.1.0

CIDR Mask: /27

Subnet Mask: 255.255.255.224

- Step 1: Number of Subnets

A /24 network is being divided into /27 subnets.

Bits borrowed:

27 - 24 = 3 bits

Number of subnets:

2³ = 8 subnets

- Step 2: Hosts Per Subnet

Host bits:

32 - 27 = 5 bits

Total addresses per subnet:

2⁵ = 32

Usable hosts:

32 - 2 = 30 hosts

- Therefore:

Number of Subnets = 8

Usable Hosts per Subnet = 30

First Three Subnets

| Subnet | Network Address | Usable Host Range | Broadcast Address. |
| --- | --- | --- | --- |
| 1 | 192.168.1.0 | 192.168.1.1 – 192.168.1.30 | 192.168.1.31 |
| 2 | 192.168.1.32. | 192.168.1.33 – 192.168.1.62. | 192.168.1.63 |
| 3 | 192.168.1.64 | 192.168.1.65 – 192.168.1.94. | 192.168.1.95 |

Q.4  You are given the IP address 10.0.5.17 with subnet mask 255.255.255.0. Determine the network address, broadcast address, and the range of valid host addresses for this subnet.<br><br><em><strong>Hint:</strong> Convert the subnet mask to CIDR notation and use bitwise AND for network address calculation.</em>?

Answer

- IP Address Calculation

Given:

IP Address: 10.0.5.17

Subnet Mask: 255.255.255.0

- Step 1: Convert Subnet Mask to CIDR

255.255.255.0 = /24

- Step 2: Network Address

Using the IP address and subnet mask:

10.0.5.17 AND 255.255.255.0

Network Address:

10.0.5.0

- Step 3: Broadcast Address

The last address in the /24 network is:

10.0.5.255

- Step 4: Valid Host Range

First usable IP:10.0.5.1

Last usable IP:10.0.5.254

- Final Answer

Network Address: 10.0.5.0

Broadcast Address: 10.0.5.255

Valid Host Range: 10.0.5.1 – 10.0.5.254

Subnet Mask: 255.255.255.0

CIDR: /24

Q.5  Use an online subnet calculator tool (such as https://www.subnet-calculator.com/) to verify your answer for the previous task. Take a screenshot of your calculation and write one line explaining if your manual answer matched the tool's output?

Answer

- My manual calculation matched the online subnet calculator output. Both showed the network address as 10.0.5.0, broadcast address as 10.0.5.255, and usable host range as 10.0.5.1 to 10.0.5.254.output.